Curb

- Stakes are referenced to top of curb and generally 3 or 7 feet to face of curb. Generally curb returns are staked on 12 offsets due to monolithic pours.

- Driveways are staked at 7 or 12 foot offsets and should be determined before staking. All driveways and handicap ramps are staked at centerline. The contractor is responsible to comply with local agency standards.

- All driveways in cul-de-sacs should be given special consideration to avoid confusion.

- All cul-de-sac curb grades should be detailed and easy to follow using an assumed stationing.

- Cul-de-sacs are the biggest cause of trouble and conflicts. It cannot be stressed enough to pay special attention when staking and referencing in this area.

- Transitions and super elevations must be clearly defined on all grading and improvement plans, as this is one of the biggest oversights.