Final Monumentation Overview

- Final monuments are generally placed in the back of the lots with iron pipes and the front of the lots with brass tags. Final monumentation should be requested by the contractor as soon as the sidewalks and fine grading are done. Final monumentation should be in before fences are installed.

- Survey monuments are generally placed in the street at intersections and beginning and end of curves and should be put in after the final lift of asphalt.

- Final monuments should be fixed in a permanent manner and display the license number of the licensed land surveyor.