Gas and Electric

- Stakes are referenced to top of curb and generally 15 or 20 feet to face of curb.

- Stakes are referenced to face of curb for gas, electric, telephone, and cable television. The depth of the trench and relationship to face of curb is the responsibility of the contractor.

- All transformers, hand holes, and streetlights are referenced to centerline and will be staked accordingly.

- All transformers or vaults should have the outside edges staked with an offset to face of curb. Line only stakes should be place behind the offset for all large objects and objects in curve.

- Cul-de-sacs need special attention to stationing, as this is a place of many conflicts and errors.  Stationing is critical and should be referenced to a point of curvature return.

- Street light locations should be cross-referenced with the improvement plan and electric plans, as this is a common place for conflicts.