Survey Stakes

4 Foot lath = Orange Painted Lath

Color Designation

- Rough Grading = Red

- Daylight Stakes = Red or Blue and White

- Finish Grade Stakes = Pink or Red

- Storm Drain = White

- Sewer = Yellow

- Reclaimed Water = Green and Blue or Green         and White

- Water Main = Blue

- Gas & Electric = White or Pink

- Walls = Green

- Bldg Corners on Non Residential = Blue or Green

2 Foot Lath = Orange Painted Lath

- Curb Stakes No Flagging

- Final Lot Corners No Flagging

- Bldg Corners for Residential Housing  No Flagging

General Notes for Points Set

- Storm Drain Inlets are generally Hub and Tack

- Building Layout Stakes are generally Hub and Tack

- Construction Staking is generally Ginny or Spike

General Notes for Grading

- // In a Slope Stake means Then

As in Then Fill or Cut

- / In a Slope Stake means And or Also

Top Construction Staking Questions Answered

- Top of Slope Stakes are 10 Offset to Top With Line Stake

- Toe of Slope Stakes are 20 Offset to Toe With Line Stake

(Toe Stakes Should Have Extra Room for the Removal at the Toe)

- Lot Lines in Grading With a Distance to the Front Property Line usually 20 & 80 to the Front PL (Property Line). (The 20 should have a grade to the front PL and both pad grades, the 80 to PL should have both pad grades with a Distance to the back Property Corner).

- When staking underground, remember an 11 foot cut should have a 15 foot offset and a 16 foot cut a 20 foot offset, etc.

5. Storm drain and Sewer is generally the deepest utility and should be examined to see which is done first.  A 15 foot offset upstream is generally the offset to the depth unless Rule 4 applies.

Sewer Laterals are generally 10 to PL with a cut to IE (Invert Elevation) at the property line.

6. Water Main is generally staked to short side on a 10 foot offset.  Grades are generally to Top of Pipe or Bottom of Pipe. Water Services are 10 to Face of curb with a cut or fill to Top of Curb. All highpoints of streets should have Air Release Valves and all Low Points should have Blowoff valves.

7. Gas & Electric is generally 15 foot to face of curb with a grade to top of curb.

8. Curb stakes are normally 3 or 7 foot offsets. Curb returns need to be 12 foot offsets when Monolithic Pours are required.

9. Final Monumentation (Property Corners) should be installed before fencing takes place.

10. Wall stakes are generally 10 foot offsets to face of wall. Face of wall should be exposed face and should be talked about for confirmation. Note: most errors occur in walls when the wall is built exactly on Property Line without compensation for the footing overhang.

General Survey Construction Staking Markings

PL = PROPERTY LINE

TOP = TOP OF SLOPE

TOE = TOE OF SLOPE

FG = FINISH GRADE

FS = FINISH SURFACE

SG = SUB GRADE

D/L = DAYLIGHT

FL = FLOW LINE

IE = INVERT ELEVATION

RIM = TOP OF RIM FINISH GRADE

TG = TOP OF GRATE FINISH GRADE

TP = TOP OF PIPE

BP = BOTTOM OF PIPE

TC = TOP OF CURB

FC = FACE OF CURB

R/W = RIGHT OF WAY

S/M = SEWER MAIN

WM = WATER MAIN

R/WM = RECLAIMED WATER MAIN

GE = GAS AND ELECTRIC

FW = FACE OF WALL

TW = TOP OF WALL

TF = TOP OF FOOTING

BF = BOTTOM OF FOOTING

C/L = CENTERLINE

L/O = LINE ONLY

BLDG = BLDG

COR = CORNER

L&D = LEAD AND DISC

SD = STORM DRAIN

FH = FIRE HYDRANT

ST. LITE = STREET LIGHT

TRANS = TRANSFORMER

TB = TOP OF BERM

CO = CLEAN OUT

AV = AIR VAC

BO = BLOWOFF VALVE

GB = GRADE BREAK

BC = BEGIN CURVE

PC = POINT OF CURVE

EC = END OF CURVE

PT = POINT OF TANGENCY

PRC = POINT OF REVERSE CURVE

PCR = POINT OF CURVE RETURN

PCC = POINT OF COMPOUND CURVE

BVC= BEGIN VERTICAL CURVE

EVC = END VERTICAL CURVE

PRVC = POINT OF REVERSE VERTICAL CURVE

CVC = CIRCULAR VERTICAL CURVE

STA. EQ = STATION EQUATION 1+00 = 9+00

D/4 = DELTA OVER 4 OR ¼ DELTA (THIS IS THE CURVE BROKEN INTO 4 EQUAL PARTS)

< = ANGLE POINT

@ = AT (USUALLY EXPRESSED FOR A DISTANCE) @ 25.00 FEET

EL = ELEVATION

© John R. Butcher & Geff Dye