Grading Overview

- Perimeter grading is the first step in grade staking and involves catching the fill and cut slopes around the perimeter of the job. Perimeter stakes help delineate the job for clearing.

- Daylight stakes are used to establish the limits of cut and fill. This is the defining line of where the grader will cut the hills and fill the canyons.

- Remedial grading is performed to ensure that slopes are placed on suitable soil foundations to prevent future slope failures.

- Rough grade stakes are used to assist in moving the dirt to its final location. The stakes can be referenced to property lines and centerline of streets.

- Finish grade stakes are the fine tune stakes to get the dirt in its final position and within tolerance as required by local agencies. The distances are given to property line on streets or common lot lines.

- Transitions and super elevations must be clearly defined on all grading and improvement plans as this is one of the biggest oversights on grading plans.

- Grade check is the final process to validate the position of slopes, pad grades, and street positions.