Sewer Overview

- Sewer systems are generally staked concentric to centerline and are referenced by station.

- Sewer systems not following centerline alignment must be detailed to allow proper location. They are graded to flow line or invert elevation and offset to the centerline of the pipe.

- Sewer laterals should follow centerline stationing and are generally stubbed to property line. It is essential to stub the sewer lateral far enough to avoid digging up concrete to find them.  An example of such is sidewalks, as shown on the video.

- Sewer laterals in cul-de-sac locations should be given extra references to help aid in proper location using a station from a known point of curb return.

- Sewer lateral tables need to be referenced accordingly with station and grades to invert elevation at property line.

- The most important thing is to verify all sewer connections before you dig.