Storm Drain Overview

- Storm Drain is usually the deepest and first utility to go in. It is essential that the job drains properly during the construction phase to avoid penalty from runoff.

- Storm drain is staked to flowline or invert elevation and to the centerline of the pipe. As with all underground staking, if there is 11 feet of cut, then a 15 foot offset should be used when staking.

- Storm drain connections must be field verified to allow proper drainage. This must be done before digging to avoid further delay and added construction costs.

- All improvements are staked at 50 foot intervals in tangent and 25 foot in curve. This includes both horizontal and vertical curves.

- All storm drain boxes should be on face of curb line produced and on string line grade with a distance to the edge of box on both stakes. These stakes should be facing each other.

- All storm drain, concentric or not, should be tied to centerline of street with station and offsets too cleanouts, inlet structures, headwalls, or any other structure.