Water Overview

- Water systems are generally staked concentric to centerline and are referenced by station.

- Water systems not following centerline alignment must be detailed to allow proper location.

- Water laterals should follow centerline stationing and are generally stubbed to the engineer’s detail. It is essential to stub the water lateral far enough to avoid digging up concrete to find them.  Fire hydrants, blow offs, and air release valves should also be stationed according to centerline.

- Water laterals in cul-de-sac locations should be given extra references to help aid in proper location.

- Water laterals are referenced to top of curb and face of curb.

- The most important thing is to verify all water connections before you dig.

- Water main is referenced to top of pipe or bottom of pipe and    should be determined before it is staked.

- Water mains using shop drawings need special consideration since everything is manufactured before it is installed. Verification of connections is very critical.